SLTM Video

Renderer

User’s guide

Revision from  03 March 2005

## ForwardT Software 3.6.0

Darim Vision Co., Ltd.

Notice

The information in this document is subject to change without prior notice in order to improve reliability, design, or function and does not represent a commitment on the part of this company.

In no event will we be liable for direct, indirect, special, incidental, or consequential damages arising out of the use or the inability to use the product or documentation, even if advised of the possibility of such damages.

Copyright © 1997 - 2005 Darim Vision Co., Ltd.

All Rights Reserved.

No part of this reference manual may be reproduced or transmitted in any form or by any means without the prior written permission of this company.

Throughout this manual, we make reference to product names that are trademarks of other companies.  We are using these names for identification purposes only, with no intention of infringement of the trademarks.

FCC Information

FCC ID:

This equipment has been tested and found to comply with the limits for a Class B digital device, pursuant to Part 15 of the FCC Rules.  These limits are designed to provide reasonable protection against harmful interference in a residential installation.  This equipment generates uses and can radiate radio frequency energy and, if not installed and used in accordance with the instructions, may cause harmful interference to radio communications.  However, there is no guarantee that interference will not occur in a particular installation.  If this equipment does cause harmful interference to radio or television reception, which can be determined by turning the equipment off and on, the user is encouraged to try to correct the interference by one or more of the following measures:

- Reorient or relocate the receiving antenna.

- Increase the separation between the equipment and receiver.

- Connect the equipment into an outlet on a circuit different from that to which the receiver is connected.

- Consult the dealer or an experienced radio/TV technician for help.

Shielded cables and I/O cards must be used for this equipment to comply with the relevant FCC regulations. Changes or modifications not expressly approved in writing by Darim Vision Co. Ltd. may void the user's authority to operate this equipment.

Limited Warranty

Our company warrants this product against defects in materials and workmanship for a period of one year from the date of purchase.  During the warranty period, products determined by us to be defective in form or function will be repaired or replaced at our option, at no charge.  This warranty does not apply if the product has been damaged by accident, abuse, misuse, or as a result of service or modification other than by us.

This warranty is in lieu of any other warranty expressed or implied.  In no event shall we be held liable for incidental or consequential damages, such as lost revenue or lost business opportunities arising from the purchase of this product.

Table of contents

1	Introduction	4

2	Use of the SLTM Video Renderer for video playback in the multipurpose Zoom Player	5

3	Use of the SLTM Video Renderer in the Graph Editor application	6

4	Filter settings dialog	7

5	SLTMVideoRendererSettings interface	9

- Introduction

SLTM Video Renderer is a Direct Show filter that allows playing back almost any video file on the FD300 board using the DirectX technology. The following formats of video compression have been successfully tested: MPEG1/2, DivX, Windows Media Video, Cinepak, Indeo and some others.

To use a filter it is necessary to install the DirectX Runtime package (it is already included in the recent versions of the Windows 98, 2000, XP operating systems). Besides, it is necessary to install a video codec for the used video format decompression. For instance to playback MPEG2-compressed video, install a MPEG decompressor, which is usually not available in Windows systems by default.

- Use of the SLTM Video Renderer for video playback in the multipurpose Zoom Player

For playing back any video file with the SLTM Video Renderer filter on FD300 board it is possible to use the multipurpose Zoom Player if the corresponding codecs for video and audio decompression are available. This shareware application can be loaded from http://www.inmatrix.com/zplayer/. The application is free for non-commercial use. Load and install it.

In a folder where the application is installed, for example, D:\Program Files\Zoom Player, create a text file named fd300.filters with any text editor, for example, Notepad, with exactly the following contents:

--------------------------------------------

LoadFilter({79D8F8C0-2C40-11D6-B6B6-0000B4907CBB},SLTMVideoRenderer)

// Since replacing the renderer breaks DirectVobSub supports,

// we need to load the DirectVobSub Automatic Filter.

LoadFilter({9852A670-F845-491B-9BE6-EBD841B8A613},DirectVobSub)

--------------------------------------------

There is a file with these contents in the SLTMVideoRenderer\ZoomPlayer folder of the current SDK. It can be simply copied to the folder where the Zoom Player is installed.

After this run the Zoom Player application, right-click its main window, select the Player Options menu item and the Manual Filters section.

The Manual Filters File list should contain the fd300 line. If this item is absent, make sure that the file described above is in the folder where the application is installed.

Enable the Using Filter File option below and select the fd300 line in the Manual Filters File list. Then press the Close button. After this any video file will be played back on the FD300 board through the SLTM Video Renderer module.

The principal Zoom Player drawback is that the application doesn’t allow opening the filter settings dialog (probably this will be implemented in future). Instead this dialog can be opened in the Graph Editor application from the Microsoft DirectX SDK package (see below), though it is less convenient in use as a player. It is possible to use this application for selecting a board in a multiboard configuration, a video format and other parameters. After that remember to enable the Automatically Save/Restore boards settings option in the filter settings dialog; see below. Later on take advantage of the Zoom Player application for more convenient playback.

- Use of the SLTM Video Renderer in the Graph Editor application

It is possible to use the Graph Editor application for playing back a video file using the SLTM Video Renderer filter. If the Microsoft DirectX SDK package is installed in the system, the Graph Editor application can be started from the system Start menu: Start–> Programs–> Microsoft DirectX8 SDK–> DirectX Utilities–> Graph Edit. It is also possible to find and run the GraphEdt.exe file manually.

After the Graph Editor application is started, perform the following operations to create a filter graph:

- Select the Graph/Insert Filters menu command. It opens a dialog with the filters categories list.

- Select the DirectShow Filters category and double-click it to open the DirectShow filters list.

- In the DirectShow Filters category select the SLTM Video Renderer filter. If this filter is missing in the list, check if it is installed correctly. The SLTMVidR.DLL file should be located in the same folder with the FD300 drivers.

- Double-click the SLTM Video Renderer filter to add it to the graph. If the application displays the message “The server threw an exception”, make sure that the FD300 board is installed and works properly. Check the system device manager and the Monitor test application operation.

- Select the File/Render Media File menu command. It opens a file open dialog to select a video file (AVI, MPEG etc.); having selected press the Open button.

- The file filter will be added to the graph. The graph manager tries to create the graph that is necessary for decompression and playback of the file with the SLTM Video Renderer filter.

- If the previous operations were performed correctly, the graph manager creates a graph with the required audio/video decompressors and combines it with the SLTM Video Renderer filter.

- Right-click the SLTM Video Renderer and select the Properties command in its local menu. This opens a dialog with several settings tabs (General, Format and Output). The first page is used for specifying the filter settings, the other two tabs are for SLTM objects of the FD300 board.

- Make sure that the status line of the General tab displays Playback on the Board#N is Enabled (where N is the number of the currently selected FD300 board; a board can be selected by the number in the Format tab). If the status line ends with Disabled or Locked instead of Enabled, video playback will not be performed.

- Make sure that the required D300 board, image size (half/full), and video standard (PAL/NTSC) are selected in the Format settings page.

- Check the settings in the Output page: video output must be set in Internal–> Analog encoder if there is no DVM digital board in the system. Signal type must be set in YUV+CVBS.

- After adjusting these settings, press the triangular Play button on the Graph Editor toolbar to start playback. Use the slider for navigation through the video file. Press the square Stop button to stop playback.

- If the image on the external TV monitor is noisy, try to invert the fields sequence: enable/disable the Upper Field First option in the General settings page. Usually this option should be disabled for standard formats such as DV, MPEG2, and enabled for the Forward board formats (ForwardJPEG, ForwardJPEG+Alpha).

- If the image is inverted vertically, enable the Vertical Invert image option on the same tab.

- If the playback is too slow (some frames are dropped), make sure that the second format in the First/Connect format field is YUY2 or Y16/Y16A. This guarantees fast conversion of video to the FD300 board video format. If the second format is RGB24/32, additional video conversion to the RGB format and back to the YUV is required, which demands significant computing power of the processor. Try using another format of the played back video material, the one compatible with YUV (some of them have been listed in the beginning of the document).

- To save and restore FD300 boards settings automatically, enable the Automatically Save/ Restore boards settings option on the filter settings tab (see below).

- Filter settings dialog

The main filter settings dialog tab is named General and contains the following controls.

The SLTM Video Renderer Settings group contains the following parameters:

- The status field displays the number of FD300 board currently selected in the Format dialog, and the current playback status, for example: Playback on the board #1 is Enabled. According to the playback status the ending of the line in the status field varies:

- Enabled – the playback on the current board is enabled;

- Disabled – the current board registration does not allow video playback in the YUV format; it is necessary to select another board or receive a correct registration key;

- Locked – some other process already plays back video on the current board; it is necessary to select another board or stop playback of the other process;

- Started – playback on the current board was successfully started.

- The Automatically Save/Restore Boards Settings checkbox enables the board settings automatic   saving/restoring. Check this checkbox to automatically save/restore all FD300 settings (video standard, output image size, etc.: all parameters of the Format and Output dialogs).

- The Vertical invert image option sets vertical invert of an image. Check this checkbox for vertical image inverting on the external TV monitor. An image can be inverted for some formats of video compression.

- The Upper field first option allows playing back the video upper field first. This option is disabled by default, which means that the video lower field is played back first. This is suitable for such compression formats as DV and MPEG2. Some other formats, for example the Forward board formats (ForwardJPEG, ForwardJPEG+Alpha) imply the reverse order of the fields. In this case enable this option to playback the upper field first. Try to switch on/off this option if on external TV monitor the image is jerky (discrete). Probably it is necessary to invert the sequence of fields. Do not enable this option if the played back materials do not contain fields (progressive scanning).

- The Autodetect fields order by compression type option detects the fields order automatically by video compression type. This option is enabled by default. If the field order is incorrect (the image on external TV monitor is discrete or noisy at playback), disable this option and try to specify the fields order manually by switching on/off the previous Upper field first option.

- The video mixer state can be specified with the Set mixer state to option. From the drop-down list select the video mixer state that will be set at playback start:

- Don’t change – do not modify the current video mixer state;

- Background – switch to the background mode (display only video scenery);

- Full mix –switch to the full mix mode (display both scenery and current video, i. e. the actor).

- The Video playback information settings group contains the following parameters:

- First/connect format is the video format identifier. This field displays two format types (FOURCCs usually): the type offered to the filter at the first CheckMediaType function execution, and the format of final connection. For the better playback it is desirable that the second format be YUY2. This guarantees fast conversion of the video to the FD300 board video format. Other connection formats possible are RGB24/32 formats, but conversion from these is slower and requires more computing loading of the processor.

- Frames played is the current number of frames played.

- Frames dropped in renderer is the current number of dropped frames. Frames may be dropped if the computer performance is not enough for decompressing a current video format in real time (at the moment of the playback).

- Average frame rate achieved is the average frames playback rate.

- Attention: only approximate frame rate is displayed.

- SLTMVideoRendererSettings interface

In addition to standard DirectShow interfaces such as the IBaseFilter, the filter has the ISLTMVideoRendererSettings interface. This interface allows controlling various filter operating modes. In fact, it allows the user tо specify the values of checkboxes in the settings dialog from the application using the filter. Some of checkboxes are not displayed in the settings dialog and can be changed only in this interface.

When the filter object is released, the settings are automatically saved in the register. When an object is created, the settings are automatically restored.

HRESULT get_AutoSaveRestore (BOOL *bAutoSaveRestore) – get the state of the automatic saving/ restoring FD300 boards settings checkbox.

HRESULT put_AutoSaveRestore (BOOL bAutoSaveRestore) –put the state of the automatic saving/ restoring FD300 boards settings checkbox. If this checkbox is checked, the settings of all FD300 boards installed in the system will be saved at deleting an object (executing the Release) and restored at creating a new object.

HRESULT get_ChangeMixerState (ULONG *nMixerState) – get the current mode of setting the output video mixer state.

HRESULT put_ChangeMixerState (ULONG nMixerState) – put the current mode of setting the output video mixer state. The following modes are possible:

- mixer_dontchange –  do not modify the state;

- mixer_background – display only background without actor;

- mixer_fullmix – display both background and actor. The specified mixer state in set before the playback is started on the selected board.

HRESULT get_InvertVertical (BOOL *bInvertVertical) – get the state of the vertical image inverting checkbox.

HRESULT put_InvertVertical (BOOL bInvertVertical) – put the state of the vertical image inverting checkbox. If the option is enabled, the image is inverted vertically at the playback.

HRESULT get_InvertFields (BOOL *bInvertFields) – get the state of the fields order inverting checkbox.

HRESULT put_InvertFields (BOOL bInvertFields) – put the state of the fields order inverting checkbox. If the option is enabled, the fields order in a frame is reversed: the upper even field goes first. The reverse order is used in the Forward board formats (ForwardJPEG, ForwardJPEG+Alpha). The direct order (the lower odd field goes first) is used in MPEG2, DV and most other formats.

HRESULT get_PlaybackStatus (LONG *nCurrentBoard, LONG *nPlaybackStatus) – get the playback status value for a board numbered nCurrentBoard to the nPlaybackStatus variable. The following modes are possible:

- playback_status_enabled – playback on the current board is enabled;

- playback_status_resourceslocked – resources are locked; probably the playback was started from another application on the current board;

- playback_status_disabled – playback is disabled by board registration;

- playback_status_started – playback is successfully started.

HRESULT get_FormatStr (LPSTR szFormatStr) – get the text string containing the FOURCCs of two video formats: the one offered to the filter at the first running (usually it is the initial video compression format, for example FRWD for the Forward board formats (ForwardJPEG, ForwardJPEG+Alpha)), and the one used for the final filter connection (for example YUY2). The function is used for information, for example this string is displayed in the filter settings dialog.

HRESULT get_PermanentYUVPlayback (BOOL *bStart) – get the state of the continuous playback checkbox.

HRESULT put_PermanentYUVPlayback (BOOL bStart) – put the state of the continuous playback checkbox. If the option is enabled, after the filters graph playback is stopped the FIFO playback of the FD300 board continues. It can be used for uninterrupted playback of a video fragments sequence.

HRESULT get_PlayWithAlpha (BOOL *bAlpha) – get the state of the video with alpha channel playback checkbox.

HRESULT put_PlayWithAlpha (BOOL bAlpha) – put the state of the video with alpha channel playback checkbox. If it is checked, the left half of frame contains a color image and the right one contains Alpha channel.

HRESULT get_RepeatFields (BOOL *bRepeat) – get the state of the fields repetition during the video playback checkbox.

HRESULT put_RepeatFields (BOOL bRepeat) – put the state of the fields repetition during the video playback checkbox. If it is checked, only the first field of a frame (even line) is used at playback, both for even and odd fields. Such mode allows playing back MPEG1-compressed video on the full screen (it is also necessary to enable the HalfSize mode for scaling half size video to the full frame size).

HRESULT get_AutoFieldsOrder(BOOL *bAuto) – get the state of the autodetect fields order checkbox.

HRESULT put_AutoFieldsOrder(BOOL bAuto) – put the state of the autodetect fields order checkbox. If the checkbox is checked, the fields order is detected automatically by the compression format type. Actually, the reverse fields order (the upper even field goes first) is used for all Forward board formats (ForwardJPEG, ForwardJPEG+Alpha), and the direct field order (the lower odd field goes first) is used for all other formats. This is correct for most standard video formats such as, for example, MPEG2 and DV. If the fields order is not detected correctly, uncheck this checkbox and specify the order manually with the help of the put_InvertFields function.