# FDTM SDK – description of the FDTM board

## Functional structure of the FDTM board

The FDTM board (FD300 series) allows to perform input and output of audio and video data, mix video and audio data, superimpose titles, and implement keying of video data.

Figure 1. Functional structure of the FDTM board

The functional parts of the FDTM board:

- The video input block provides simultaneous input of up to two video streams from analog or digital input and keying of one of the video streams.

- The video output block forms standard analog video signal from video data stream from the output of video mixer or second digital bus. The input video signal can be genlocked (synchronized) with any input video signal.

- The video mixer block provides mixing of “Artist” image from the video inputs and computer images “Scenes” from PC, titles superimposing, etc.

- The Connection with PC block provides transferring video and audio data streams to/from PC. Moreover, the video data input can be from two sources – from the board’s input (“Artist”) and from the mixer output (“Output”).

- The audio input block allows to input up to 6 audio channels.

- The audio output block allows to output up to 6 audio channels.

- The audio mixer forms delay of audio data (for arbitrary time from 0 to 999 milliseconds regardless for each channel), forms the output audio streams from a random combination of all input audio streams and audio data streams from PC.

The following signals sources/receivers can be connected to the inputs and outputs of the board:

- Up to 12 video sources of composite signal (VHS), up to 6 video sources of S-Video (S-VHS), and up to 3 video sources of component video signal of RGB YCrCb, RGB+Sync or YCrCb+Sync format can be connected to the analog video input. In this case no more than 2 video sources can be used simultaneously, i.e. the board allows the user to select a video source to work with at the moment, as there is no need for physical re-switching.

- The receiver of composite, S-Video or component video signal can be connected to the analog video output.

- Special extension boards, e.g. the DVM board to form TV video signal from the output of a standard VGA card, can be connected to the digital buses.

- Up to 6 audio sources (e.g. 3 stereo sources) and 6 audio receivers correspondingly can be connected to the audio inputs and outputs.

## Transfer (mixing) scheme of audio data of the FDTM board

The FDTM board provides input and output of 6 audio channels (i.e. 6 mono inputs and 6 mono outputs). These inputs and outputs can be joined in pairs to stereo inputs and outputs (i.e. 3 stereo inputs and 3 stereo outputs). The transfer of audio data from any mono input to any mono output with various volume and delay is implemented within the board.

Figure 2. Transfer (mixing) scheme of audio data of the FDTM board

Every mono input Mono Input N and mono output Mono Output N have hardware regulator (controller) of volume and switch (mute). Moreover, the pairs of inputs Mono Input 2 + Mono Input 3 and Mono Input 4 + Mono Input 5 have their own switches of extra gain for 20dB (mic mode).

The Cross Mix module represents a connection of each input with every output. Its own volume and its own transfer audio switch (mixing) is set for every connection of i input with j output. The value of audio delay is set for each input individually within the range from 0 to 999 milliseconds.

From the point of view of operation system the FDTM board represents tree standard WAVE-devices, namely three WaveIn devices (to input audio) and three WaveOut devices (to output), every device operates with its stereo channel. The audio from PC incoming to the first WaveOut is added to Stereo Output 0. Therefore, audio from the second WaveOut device feeds Stereo Output 1, from the third one - Stereo Output 2. Unlike the clear connection of output audio devices, the WaveIn devices can input audio from any stereo input.

## Hardware scheme of the FDTM board

The FDTM boards have two input decoders, output coder, three audio codecs, FPGA Spartan, DSP TriMedia, and several memory chips for buffering input video streams, storing two keying tables and memory for DSP TriMedia:

Figure 3. Hardware scheme of the FDTM board

Every video decoder captures and decodes video data from any analog video source or from wide (40-pin) digital bus (656 bus + IIC bus) and transfers the data to FPGA Spartan. Moreover, every video decoder transfers synchronization signal to FPGA Spartan, which can be used by the output coder for genlocking.

FPGA Spartan deals with video data buffering and keying of one of video streams, as well as controls the synchronization (genlocking) modes.

The output coder forms analog video signal in CVBS format (composite video signal), S-Video or YCrCb/RGB/YCrCb+Sync/RGB+Sync (component video signals). The coder can receive source digital video data from DSP TriMedia or an other device via narrow (26-pin) digital bus (656 bus) (e.g. from DVM board).

TriMedia receives the stream of packed video data from FPGA Spartan and outputs the video data stream to the output coder. TriMedia in a special mode can send increased (enlarged) video data stream to the video coder, adding to it information about Alpha-channel (mask). In this case the output video coder forms video signal in the YCrCb + Mask format with synchro-mix addition to the Y signal or Mask signal.

The FDTM board has three audio codecs, each of them provides one stereo input and one stereo output (totally 6 mono inputs and 6 mono outputs). All of them enter to FPGA Spartan via individual IIS bus; FPGA Spartan combines and transfers audio data to/from TriMedia. The application in TriMedia in its turn provides data transferring from any input to any output with various volume and delay.

## Functional scheme of FPGA Spartan

The following items are implemented in FPGA Spartan:

- Asynchronous recording to video data memory from the first and second decoders (full frame is buffered – two fields);

- Synchronous reading both video streams from the memory and their commutation (switching) (e.g. the use of video data from the second decoder as an active channel);

- Active channel keying (receive an image with color correction with Alpha-channel from the current keying table);

- Formation of data stream for transferring to TriMedia.

Figure 4. Internal structure of FPGA Spartan video streams

The frames FIFO serves for decoupling video frequencies of two video sources (input video decoders) and output video coder of the FDTM board, i.e. for implementation of signals synchronization (TBC – Time Base Correction). FPGA starts recording to appropriate FIFO upon the signal of “frame start” from the decoder. FPGA marks the stored address as a “correct” one upon the frame end from this decoder; this address will be used while reading following frame from FIFO. For normal internal logic operation FPGA requires that at least once a “correct” full frame will be from every decoder

Two selectors allows to change the decoders usage order, precisely to use the second decoder as an active channel, i.e. video data source for keying and color correction. The output part serves to form a packed video data stream of 16 pixels for transferring to DSP TriMedia (refer to section 6 for the detailed information about video data format in FIFO).

## Functional scheme of DSP TriMedia

TriMedia receives a stream of packed video data from FPGA Spartan and outputs the video data stream to the output coder:

Figure 5. Internal structure of TriMedia video streams

TriMedia contains several software objects: Artist, Mixer, Output, Scene, Grabber 1, and Grabber 2.

- The “Artist” object receives video data from FPGA Spartan and stores them in the TriMedia memory (two buffers for even and odd fields).

- The “Scenes” object reads video data from PC memory to its buffers.

- The “Mixer” object inquires the addresses of memory buffers with corresponding fields of the artist and scene and mixes the image to its memory (also to two fields), which later sends to the “Output” object for displaying.

- All operation of video data mixing is synchronized by the “Output” object, which receives interrupts from the video decoder. When it receives next interrupt, the “Output” object inquires the mixer about the buffer address for displaying. Having returned the address of a current buffer, the mixer starts mixing subsequent field, it inquires the Artist and Scene about buffers addresses. Artist switches the buffer address for reading on input interrupts and doesn’t depend upon the mixer (and, therefore, upon the “Output” object). Scene in its turn as soon as it gives the address of a current buffer starts reading next filed from PC memory.

NOTE: If you do not have output interrupts, e.g. if you have selected narrow bus (26-pin) and there is no signal on it, the process of data mixing will stop! Scene will also stop reading data, and the second grabber will stop recording data. The first grabber may also stop data input, if the “Master” mode is selected as a source of input frequency, i.e TriMedia output bus.

- Two “Grabber” objects are realized in TriMedia: the first grabber is to input video data to PC from input, and the second grabber – from mixer output. Though only one object to input data (the ISLTMGraber interface) can be created in the SLTMSrv object, you can create two SLTMSrv objects, assign them to the same FDTM board and one of them should start data input from the input, the other one from the output. Note, that there is only one input video data stream, i.e. both input streams (from Decoder A and Decoder B) and Alpha-channel stream are transferred by one data stream in a special packed format, convenient for processing and mixing by the TriMedia processor. In both cases data are input in fifo format, described in section 6.

## Functional scheme of DSP TriMedia Mixer

TriMedia mixer can mix up to 5 video data streams:

Figure 6. Structure of TriMedia mixer

Input selector can connect video data streams from FD300 video inputs or from host PC to 4 mixer lines: Background, Mask, Foreground and Title. Background and Foreground is video data stream with format YUV 4:2:2. Mask is alpha data stream, it is possible put here alpha mask from keying. Title is complex data stream, which includes video and alpha data (usually with special RLE compression – YUVA 4:2:2:4). First internal mixer combines Foreground and Background images by Mask. Second mixer overlays title image with alpha channel on result image from first mixer. It is possible disable any mixer line:

- Background or Foreground only,

- mix Foreground and Background by Mask,

- overlays titles on Background or Foreground,

- overlays titles on Foreground and Background mixed by Mask.