SLTMTitleRunTime SDK – a table of contents

SLTMTitleRunTime SDK – brief description	2

How to create projects with SLTMTitleRunTime SDK	3

SLTMTitleRunTime SDK – objects	4

SLTMTitleRunTime SDK – interfaces	5

ISLTMTitleRunTimeMgr	5

ISLTMTitleRunTimeMgr2	5

ISLTMTitleRunTime2	6

The appendix	9

Example: playback of title project	9

Possible problems at opening титровального the project	10

# SLTMTitleRunTime SDK – brief description

The package of software developer SLTMTitleRunTime SDK is intended for third-party developers who creates application with title usage on FD300 board. The package allows to open project file, which was created by FDTitleDesigner application, to list all created in the project title objects and to operate each of objects independently from each other:

- load new task into the object,

- start playback of the task,

- stop playback (softly or immidately),

- continue playback from interrupted position,

- estimate time to real stop,

- estimate duration of other task (without loading it into the object).

Everyone title object at every time can have and playback only one task. New task loading automatically stops current playback. The title objects number theoretically is not limited, but practically it is limited by accessible resources (physical memory and CPU power).

Attention! For use SLTMTitleRunTime SDK it is necessary to FD300 board software with necessary version corresponding to version SDK (for example if the folder refers to SLTMTitleRunTimeSDK_300 it is necessary to install ForwardTSoftware versions 3.00 or is higher).

SLTMTitleRunTime SDK will consist of the following parts:

| The name of a part | Where is |
| --- | --- |
| Programs and services | Are installed FD300 board software installer |
| Included header files (h-files) and source files (С-files) with constants and intefaces definitions | Folder [Includes] Should be included in the created project in language C, see «How to create projects with SLTMTitleRunTime SDK» |
| The documentation – the objects and interfaces description | Folder [Docs] |

The description of the basic objects and interfaces SLTMTitleRunTime SDK is resulted in the given document.

# How to create projects with SLTMTitleRunTime SDK

Program modules SLTMTitleRunTime SDK represent the COM-objects installed and registered in system at FD300 board software installation. As for a call of objects technology COM is used, in the beginning of work the application have to call CoInitializeEx(NULL, COINIT_MULTITHREADED) in the beginning of each of executed thread, otherwise any next call or object creation will return error code «COM library is not initialized». Accordingly, at end of work of each thread it is necessary to calle CoUnitialize().

In any case, it is recommended to create projects in Microsoft Visual Studio environment of version 6.0 or is higher, with ATL library usage. At use of the given library initialization COM will be inserted into the project automatically. Besides ATL gives a set of functions and the classes considerably facilitating development of COM-projects (in particular, class CComPtr – so-called smart pointer, automatically calculated number of references to the object).

If the program project is created as ATL Project then ATL support included automatically. However, it can be demanded to create MFC Project which don’t support ATL by default. To include ATL support into already created MFC-project, press the right button of the mouse above a tree of classes of the project and choose item “New ATL Object”. The question will appear: «Do you want to connect support ATL in your MFC-project?». Answer "Yes" the question and press "Cancel" in the following dialogue of addition ATL-object creation (if it will appear). The files necessary for ATL support will be automatically added in the project.

To use SLTMTitleRunTime SDK objects in the created project with ATL support it is necessary:

- Copy entirely folder [Includes] with all included h- and С-files into folder with the project.

- Include file SLTMTitleRunTime.h in the basic heading h-file of the project and file SLTMTitleRunTime_i.c – in the basic C-file of the project (#include "SLTMTitleRunTime.h", #include "SLTMTitleRunTime_i.c").

- For convenience it is possible to include IDL-files from folder [IDL] with definitions of interfaces in the project that names of interfaces and methods were displayed in project window of Visual Studio. Do not fogot to turn on flag «Exclude file from Build» for all IDL-files because of thay should not compiled – for this purpose in a project folder “File View” add files from a folder [IDL], choose all added files, press the right button of the mouse and in dialogue of properties of a file switch off the specified flag.

At work with other compilers (for example, Borland Builder) it is necessary to import classes from file SLTMTitleRunTime.tlb usually located in a folder "C:\Program Files\ForwardT Software\FDTitle\".

# SLTMTitleRunTime SDK – objects

The given section lists all basic objects of SLTMTitleRunTime SDK. For each object it is specified his interfaces. Detailed description of the interfaces are resulted in section «SLTMTitleRunTime SDK – interfaces».

coclass TitleProjectMgr

{

interface ISLTMTitleRunTimeMgr;

interface ISLTMTitleRunTimeMgr2;

};

Object TitleProjectMgr allows initialize title engine, load title project and give access to each title object of the project.

# SLTMTitleRunTime SDK – interfaces

## ISLTMTitleRunTimeMgr

Object: CLSID_TitleProjectMgr

The interface allows initialize title engine, load title project (created by FDTitleDesigner application) and give access to each title object of the project.

HRESULT Init([in] BSTR fileName, [in] long boardIndex) – initializes title engine on FD300 board with index “boardIndex” and creates all title objects described in the project with file name “fileName”. At project loading title engine can find the problems and will show message about it. In this case the given method will return error code E_INVALIDARG. For obtaining number of FD300 boards it is necessary to use another SDK, for example VideoPalyer SDK.

HRESULT Uninit() – release all created title objects and all title resources on FD300 board.

HRESULT get_NumObjects([out, retval] long* pRetVal) – returns number of created title objects.

HRESULT GetName([in] long index, [out, retval] BSTR* pRetVal) – returns a name of title object with specified index (from 0 up to object number - 1).

HRESULT GetRunTimeInterface([in] long index, [out, retval] IUnknown ** pRetVal) – returns the reference on title object (interface ISLTMTitleRunTime2) with the specified index for the further object manipulation.

## ISLTMTitleRunTimeMgr2

Object: CLSID_TitleProjectMgr

The given interface is completely similar to interface ISLTMTitleRunTimeMgr with one exception – procedure of initialization (project loading) has the additional parameter which allow to specify GUID of the title engine (see notes 1):

HRESULT Init([in] BSTR fileName, [in] long boardIndex, [in] GUID* targetGuid) – initializes title engine for the object with GUID “targetGuid” (see the appendix 1) and, probably, with index “boardIndex”, and then creates all title objects described in the project with file name “fileName”

HRESULT Uninit() – release all created title objects and all title resources on FD300 board.

HRESULT get_NumObjects([out, retval] long* pRetVal) – returns number of created title objects.

HRESULT GetName([in] long index, [out, retval] BSTR* pRetVal) – returns a name of title object with specified index (from 0 up to object number - 1).

HRESULT GetRunTimeInterface([in] long index, [out, retval] IUnknown ** pRetVal) – returns the reference on title object (interface ISLTMTitleRunTime2) with the specified index for the further object manipulation.

Notes:

- Now exist following title engines:

CLSID_SLTMTitleServerTargetMgr – title engine for input “Title” of FD300 board mixer (“boardIndex” means index of FD300 board).

CLSID_SLTMTitleArtServerTargetMgr – title engine for input “Foreground” of FD300 board mixer (“boardIndex” means index of FD300 board).

CLSID_SLTMTitleBackServerTargetMgr – title engine for input “Backgound” of FD300 board mixer (“boardIndex” means index of FD300 board).

## ISLTMTitleRunTime2

The given interface allows manipulate by title object (see note 1). It allows to load title task into the object, start/stop playback, estimate time upto real playback stop (for current task or another task) and so on. The interface pointer can be received from object TitleProjectMgr by GetRunTimeInterface method.

HRESULT get_Name([out, retval] BSTR* pRetVal) – returns name of the title object.

HRESULT get_Status([out, retval] SLTMRunTimeStatus* pRetVal) – returns the current state of the title object (see note 2).

HRESULT Start([in] double tStart, [in] double tDuration) – start playback of the loaded task from current position. Playback will started at time “tStart” (see note 3). Title object will appear smoothly from the playback start moment during “tDuration” (in seconds).

HRESULT Stop([in] double tStart, [in] double tDuration) – will stop current playback upto time “tStart” (see note 3). If by this time the next unbreakable part of current title tasks (one graphic or animated file) will not be finished, then this part will not included into current playback. If command Stop is issued too late and the next part of the task already started to playback, then the playback will stopped later (after time “tStart”). For this situation detection it is recommended to use method GetPlayInfo of the interface. By the moment of playback ending during time “tDuration” (in seconds) the title object will smoothly disappear (becomes completely transparent).

HRESULT Abort([in] double tDuration) – immediately stops current playback. Thus the title object will start playback from the unbreakable part which was not shown completely. By the moment of playback ending during time “tDuration” (in seconds) the title object will smoothly disappear (becomes completely transparent).

HRESULT get_TaskFileMask([out, retval] BSTR* pRetVal) – returns standard file dialog  mask to choice tasks fro the object, for example «Image files (*.tga; *.jpg; *.png; *.bmp)|*.tga; *.jpg; *.png; *.bmp|» or «Movie Files (*.avi)|*.avi|Project Files (*.tml)|*.tml|».

HRESULT LoadTask([in] BSTR file) – loads the task from a file “file” into the title object. The command atomatically stop current playback.

HRESULT get_TaskFileName([out, retval] BSTR* pRetVal) – returns a name of the current task.

HRESULT get_Loopped([out, retval] long* pRetVal) – returns a loopped flag for the current task (see note 4).

HRESULT put_Loopped([in] long newVal) – sets a lopped flag for the current task (see note 4).

HRESULT GetTaskInfo([in] BSTR fileName, [out] long* pLoop, [out, retval] double* pDuration) – returns parameters of the task from file “filename” if it has been loaded into title object right now: “pLoop” – the task is lopped or not, “pDuration” – duration of the task.

HRESULT GetPlayInfo([out] double* currTime, [out] double* restTime, [out] double* allDuration, [out] long* Loopped) – returns the information about current playback: “currTime” – current time inside title engine, “restTime” – rest time to finish playback of current unbreakable part of the task, “allDuration” – rest time to finish playback of whole current task, “Loopped” –is looped the current task or not (see notes 3,4).

Notes:

- Title object is the group in FDTitleDesigner application. The group can consist from one or few title elements, such as RollCrawl, TV Clock, Movie or Picture with transparency. If the group (title object) includes only one title element then the task for such title object is the task for the element, for example, AVI-file for an element «Movie with transparency». If the group includes few title elements, for example, a crawl and a it’s background picture as “Picture with transparency” then title task for such title object will be a file created by command “Save task” of FDTitleDesigner application. At start/stop playback of such title object all title elements will start and stop playback together.

- Possible state of title object (SLTMRunTimeStatus):

SLTMRunTimeStatus_eNone – the object is not initialized.

SLTMRunTimeStatus_eStopping – playback is stopping but is not stopped yet. For example, last unbreakable part of the task is played.

SLTMRunTimeStatus_ePreparing – the object is preparing for new task playback.

SLTMRunTimeStatus_eStopped – playback was stopped (or task is not loaded).

SLTMRunTimeStatus_eStarting – playback is starting (object is smoothly appear).

SLTMRunTimeStatus_eRunning – the object is played normally.

The possible state transition is presented on folloing picture (except SLTMRunTimeStatus_eNone):

  - Usually the object is in state SLTMRunTimeStatus_eStopped.

  - At command LoadTask the object can pass in state SLTMRunTimeStatus_ePreparing (while it prepares for future playback of the new task) and then comes back in state SLTMRunTimeStatus_eStopped.

  - On command Start the object goes in state SLTMRunTimeStatus_eStarting and at specified time goes in state SLTMRunTimeStatus_eRunning.

  - On command Abort the object goes in state SLTMRunTimeStatus_eStopping and after the real disappearance from the screen it goes in state SLTMRunTimeStatus_eStopped.

  - On command Stop the object remembers specified time and goes in of a stop SLTMRunTimeStatus_eStopping during playback of the last unbreakable part of the task before specified time. Aftre that the object goes in state SLTMRunTimeStatus_eStopped. Attention: title object can be long time enougph in state SLTMRunTimeStatus_eStopping.

- The parameter tStart of Start/Stop commands sets moment of command performance. If value “tStart” is negative it means that the command should be executed through time abs(tStart). If value “tStart” is zero the command should be executed as soon as possible ("right now"). If value “tStart” is positive it means the global time of title engine which can be obtained by method GetPlayInfo of given interface (the first returned parameter).

- If the task is looped then the title object automatically start playback from the beginning at task finish. If the task is not looped then of task playback finish the title object goes in state SLTMRunTimeStatus_eStopped. In this case for further task playback it is necessary to load again this or other task into title object.

- Title tasks generally consist from several parts. For example, a crawl script consist from separate graphic files. Playback of one graphic file is unbreakable part of the title task – it cannot be interrupted by command Stop. Therefore for normal title object playback (for a correct stop and start of playback, and also for loading the new task on whole task playback finish) it is necessary to obtain information both on whole task finish and current unbreakable part finish. Method GetPlayInfo returns in the second parameter “restTime” rest time to finish playback of current unbreakable part, and in the third parameter rest time to finish playback of whole task. Attention: after stop and continuation playback of the task rest duration can be changed because of in normal playback all files played consequently but on every break at least one “clear” screen appear.

# The appendix

## Example: playback of title project

Object TitleProjectMgr allows load title project which was created by FDTitleDesigner application and give access to each title object of the project. For each object it is possible to start/stop playback, load new task and so on. Lets try to make following steps:

- Create object TitleProjectMgr.

- Load existing title project.

- Start synchronous playback of all title objects.

- Wait 30 seconds.

- Stop playback step by step for all title objects with one second delay.

Below it is given an example the procedure which makes all listed operations (the full path to the title project is parameter “pstrFileName”, duration of title fade in and fade out is parameter “duration”):

Strongly recommended to check result of each operation such as HRESULT with the help macro FAILED, and to stop work if the call of any method has returned a error.

#import "SLTMTitleRunTime.tlb"

HRESULT PlayProject(LPCTSTR pstrFileName, double duration)

{

HRESULT hr;

// Create object TitleProjectMgr

SLTMTitleRunTime::ISLTMTitleRunTimeMgr2Ptr pIProject;

hr = pIProject.CreateInstance(__uuidof(SLTMTitleRunTime::TitleProjectMgr));

if (FAILED(hr)) return hr;

try {

// Will use title engine for “extermal mixer” mode

GUID guid = CLSID_SLTMTitleNoPremultServerTargetMgr;

// Load the project in FD300 board with index 0

_bstr_t bstrName(pstrFileName);

pIProject->Init(bstrName,0,&guid);

// Get access to first title object (with index 0) ...

SLTMTitleRunTime::ISLTMTitleRunTime3Ptr pIObject;

pIObject = pIProject->GetRunTimeInterface(0);

// ... and obtain current board time

double fStartTime = 0;

double tmp1;   long tmp2;

pIObject ->GetPlayInfo(&fStartTime, &tmp1, &tmp1, &tmp2);

// Release the object

pIObject = NULL;

// Shift time of playback start forward for half-second

fStartTime += 0.5;

// Obtain number of title objects

long number = pProject->NumObjects();

// Cycle by title objects

for( int i=0; i<number; i++ )

{

// Receive reference to title object with index ‘i’

SLTMTitleRunTime::ISLTMTitleRunTime3Ptr pIObject;

pIObject = pIProject->GetRunTimeInterface(i);

// Start playback at absolute time fStartTime

pIObject->Start(fStartTime,duration);

// Release the object

pIObject = NULL;

}

// Wait, for example 30 seconds

sleep(30000);

// Цикл по всем титровальным объектам

for( int i=0; i<number; i++ )

{

// Receive reference to title object with index ‘i’

SLTMTitleRunTime::ISLTMTitleRunTime3Ptr pIObject;

pIObject = pIProject->GetRunTimeInterface(i);

// Stop playback through one second

pIObject->Stop(-1.0,duration);

// Release the object

pIObject = NULL;

}

// Wait one second

Sleep(1000);

// Release resources of FD300 board

pIProject->Uninit();

}

catch(_com_error& err) {

// was error – error code = err.Error() – except special error code

// which means that run-time showed message box before

if( err.Error() != (HRESULT)(0x81234567) )

MessageBox(m_hWnd, err.Description(), _T("TestSDK"));

}

// all finished

return S_OK;

}

Attention!!! Pragma #import… declare special classes which can be used instead of CComPtr<> except one – the classes generate exceprtion on failed HRESULT of COM-object method call.

## Possible problems at opening титровального the project

At title project loading the following problems are occures:

- Into specified title engine was already created title objects which are placed on the same layers as new loading objects. For the decision the problem it is necessary to set different values “Base Z order” in FDTitleDesigner application for different title objects (groups) in all used projects, for example 110, 120, 130... for one project and 210, 220, 230... for other project.

- Invalid property warning messages usually means inconsistency of software versions (version of title engine) which are used now and which was used for title project creation. In this case it is recommended to open the project in FDTitleDesigner application, set invalid values and save the project again.