VideoPlayer SDK – a table of contents

VideoPlayer SDK – the brief description	2

How to create projects with use VideoPlayer SDK	3

VideoPlayer SDK – objects	4

VideoPlayer SDK – interfaces	5

ISLTMVideoPlayer	5

ISLTMLive4VideoPlayer	7

ISLTMAudio4VideoPlayer	8

ISLTMForegroundPlayer	10

ISLTMAudioPlayer	12

The appendix	14

Example: reproduction of sequence of AVI-files with use of object SLTMVideoPlayer	14

# VideoPlayer SDK – the brief description

The package of software developer VideoPlayer SDK is intended for a spelling of the appendices using opportunities of payments FD300 for reproduction of video and a sound.

Attention! For use VideoPlayer SDK it is necessary to establish ON payments FD300 of the necessary version corresponding to version SDK (for example if the folder refers to VideoPlayerSDK_300 it is necessary to establish ON ForwardTSoftware versions 3.00 or is higher).

VideoPlayer SDK will consist of the following parts:

| The name of a part | Where is |
| --- | --- |
| Executed program modules and services | Are established in structure of the software of payment FD300 |
| Included h-files of headings and С-files with definitions of constants and identifiers of classes of objects and interfaces | Folder [Includes] Should be included in the created project in language C, see «How to create projects with use VideoPlayer SDK» |
| The documentation – the description of program objects and interfaces | Folder [Docs] |

The description of the basic objects and interfaces VideoPlayer SDK is resulted in the given document.

# How to create projects with use VideoPlayer SDK

Program modules VideoPlayer SDK represent the COM-objects established(installed) and registered in system at installation of drivers and ON payments FD300. As for a call of objects technology COM is used, in the beginning of work the causing appendix is obliged to cause function CoInitializeEx (NULL, COINIT_MULTITHREADED) in the beginning of each of executed streams of management, otherwise any subsequent call (and even) will return creation of a copy of object an error code « library COM » is not initialized. Accordingly, at end of work of each stream it is necessary to cause CoUnitialize ().

In any case, it is recommended to create projects in Microsoft Visual Studio environment of version 6.0 or is higher, with use of library ATL. At use of the given library initialization COM will be inserted into the project automatically. Besides ATL gives a set of functions and the classes considerably facilitating development of COM-projects (in particular, class CComPtr – so-called “ the clever index ”, or smart pointer, automatically tracing quantity(amount) of references to object).

If the program project is created as ATL Project support ATL is included in him(it) automatically. However, it can be demanded to create MFC Project in which by default support ATL is not included. To include her(it) in already created MFC-project, press the right button of the mouse above a tree of classes of the project, and choose item(point) “ New ATL Object ”. The question will appear: « Do you want to add ATL support to your MFC project? » (whether « you Want to connect support ATL in your MFC-project? »). Answer "Yes" the set question, and press "Cancel" in the following dialogue of addition of ATL-object (if it(he) will appear). The files necessary for support ATL, will be automatically added in the project.

To use objects VideoPlayer SDK in the created project with support ATL, it is necessary:

- To copy entirely a folder [Includes] with all included h-and С-files in a folder where there is a project.

- To include file VideoPlayerSDK.h in the basic heading h-file of the project, and file VideoPlayerSDK_i.c – in the basic C-file of the project (*include "VideoPlayerSDK.h", *include "VideoPlayerSDK_i.c").

- For convenience it is possible to include IDL-files from a folder [IDL] with definitions of interfaces in the project that names of interfaces and methods were displayed in a window of project Visual Studio. do not overlook to include a flag « Exclude file from Build » for all IDL-files since to be compiled they should not – for this purpose in a bookmark “ File View ” the project add files from a folder [IDL], choose all added files, press the right button of the mouse and in dialogue of properties of a file switch off the specified flag.

# VideoPlayer SDK – objects

In the given section all basic objects given VideoPlayer SDK are listed. For each object it is specified, it gives what interfaces. Interfaces and the detailed description of their methods are resulted in following sections.

coclass SLTMVideoPlayer

{

interface ISLTMVideoPlayer;

interface ISLTMLive4VideoPlayer;

interface ISLTMAudio4VideoPlayer;

};

Object SLTMVideoPlayer is intended for video & audio playback. Interface ISLTMVideoPlayer allows to be switched between computer and LIVE video (video from the current input of a FD300 board) by transparency mask (crossfade). Interface ISLTMAudio4VideoPlayer allows control audio parameters of FD300 board (loudness and mute), used at audio playback together with video playback.

coclass SLTMForegroundPlayer

{

interface ISLTMForegroundPlayer;

};

Object SLTMForegroundPlayer is intended for playback "old style" titles by interface ISLTMForegroundPlayer. It does not allow playback of audio.

coclass SLTMAudioPlayer

{

interface ISLTMAudioPlayer;

};

Object SLTMAudioPlayer is intended for playback of audio files by interface ISLTMAudioPlayer.

# VideoPlayer SDK – interfaces

## ISLTMVideoPlayer

Object: CLSID_SLTMVideoPlayer

The given interface is intended for playback of video. It allows to be switched between computer video and LIVE video (from the current input of FD300 board) with crossfade.

HRESULT get_BoardsCount ([out, retval] long *pVal) – returns value of property BoardsCount: quantity of FDTM boards, which was installed in the system.

HRESULT get_BoardName ([in] long BoardIndex, [out, retval] BSTR *pName) – returns the text name of FDTM board with specified index BoardIndex.

HRESULT UsedMemorySizeMB ([in] long use_MB) – sets size of memory for buffering the video data (recommended value: 40). Attention! This function should be called before AcquireBoard call.

HRESULT AcquireBoard ([in] long BoardIndex) – tries to lock resources for video playback on specified FD300 board (with index BoardIndex, numbering from zero). Attention! At playback of video with audio it is necessary to choose before playback audio device by function SelectDevice of interface ISLTMAudio4VideoPlayer.

HRESULT FreeBoard () – frees the locked resources.

HRESULT GetBoardInfo ([out] long *width, [out] long *height, [out] double *frame_rate) – returns the current parameters of the selected board: width – width of the screen (in pixels), height – height of the screen (in pixels), frame_rate – frequency of the video (frames per second).

HRESULT get_AutoScaleImage([out, retval] long *pEnabled) – returns 1 if static image auto scale mode eneabled.

HRESULT put_AutoScaleImage([in] long bEnable) – enables/disables image auto scale mode.

HRESULT get_AutoScaleMovie([out, retval] long *pEnabled) – returns 1 if static movie auto scale mode eneabled.

HRESULT put_AutoScaleMovie([in] long bEnable) – enables/disables movie auto scale mode.

HRESULT ShowLive ([in] double start_time, [in] double crossfade_duration) – will switch the FD300 internal video-mixer on display of LIVE video (from a currently selected video input) at specified global time with the specified crossfade duration (in seconds). See the note 1.

HRESULT LoadImage ([in] long image_index, [in] BSTR file_name) – loads and prepares for playback a picture from the specified graphic file (a picture in format JPEG, Bitmap, Targa or PNG). To a picture the numerical index image_index will be given (can be 0 or 1) on which it can be chosen for playback later (it can be loaded up to 2 pictures).

HRESULT ShowImage ([in] long image_index, [in] double start_time, [in] double crossfade_duration) – will show the picture loaded earlier with index image_index at specified global time start_time with specified crossfade duration (in seconds). See the note 1.

HRESULT PrepareVideo ([in] BSTR file_name, [out] double *duration) – loads and prepares for playback an AVI-file with specified file name. Duration of prepared video will be returned in parameter duration.

HRESULT PrepareVideoOnly ([in] BSTR file_name, [out] double *duration) – loads and prepares for playback an AVI-file with specified file name without playback of audio from the specified file. Duration of prepared video will be returned in parameter duration.

HRESULT PrepareVideoPart ([in] BSTR file_name, [in] double fMarkIn, [in] double fMarkOut, [out] double *duration) – loads and prepares for reproduction an AVI-file with specified file name. fMarkIn is time of first visible frame, fMarkOut is time of first invisible frame. Duration of prepared video will be returned in parameter duration.

HRESULT PrepareVideoPartOnly ([in] BSTR file_name, [in] double fMarkIn, [in] double fMarkOut, [out] double *duration) – loads and prepares for reproduction an AVI-file with specified file name without playback of audio from the specified file. fMarkIn is time of first visible frame, fMarkOut is time of first invisible frame. Duration of prepared video will be returned in parameter duration.

HRESULT ResetPrepare () – releases the resources seized by playback preparation (closes the file, frees a buffers).

HRESULT ShowPreparedVideo () – shows the first frame of prepared video.

HRESULT PlayVideo ([in] double start_time, [in] double crossfade_duration) – will start playback of prepared video at specified global time start_time with specified crossfade duration (in seconds). See the note 1.

HRESULT PauseVideo () – pause playback of video (still show current frame).

HRESULT UnpauseVideo () – continues playback of paused video.

HRESULT get_RestDuration ([out] double *pEndingTime, [out, retval] double *pRestDuration) – returns planned time of the playback finish into parameter pEndingTime and time up to the end of crossfade finish into parameter pRestDuration (in seconds). See the note 1.

HRESULT get_PrepareState ([out, retval] long *pVal) – returns the current state after last command PrepareVideo. See the note 2.

HRESULT get_ErrorDescription ([out, retval] BSTR *pVal) – returns a string with the text description of the error of last video preparing.

HRESULT get_PlaybackState ([out, retval] long *pVal) – returns the current state after last command PlayVideo.

HRESULT get_PlaybackErrorDescription ([out, retval] BSTR *pVal) – returns a string with the text description of the playback error.

Notes:

- Global time (in seconds) is an internal time of FD300 board. It starts from the FD300 board initiation. Crossfade – transition from one video-image on another through the cross-fade effect.

- Video player executes playback preparing commands asynchronously. It means that your code will continue execution before real preparing of video data. Therefore you need to use function get_PrepareState to detect a finish of video preparing on commands PrepareVideo or PrepareVideoOnly. In this case the following states (SLTMVideoPlayerPrepareState) are possible:

SLTMVideoPlayerPreparingVideo 	video preparing is not finished, video playback is not ready, you can not call command PlayVideo.

SLTMVideoPlayerPreparedVideo 	video preparing is finished, video playback is ready, you can start playback by command PlayVideo.

Further, after a call of function PlayVideo begins a transition from previously played video to already prepared. Therefore, after a call of function PlayVideo the following states are possible:

SLTMVideoPlayerInTransition 	transition (crossfade) executing, you cannot start playback or preparing on next video.

SLTMVideoPlayerStandby 	transition is finished, playback of new video goes (or is already finished) and it is possible to start a next video preparing.

If an error happens during preparing then function get_PrepareState will return SLTMVideoPlayerWasError. In this case it is necessary to call function ResetPrepare, and before it it is possible to receive the text description of the error by function get_ErrorDescription.

If an error happens during playback then function get_PlaybackState will return SLTMVideoPlayerWasError. In this case it is possible to receive the text description of the error by function get_PlaybackErrorDescription.

- Supported formats of audio and video files are listed in the description of interface ISLTMPlayMovie, and formats of graphic images – in description ISLTMPlayImage.

## ISLTMLive4VideoPlayer

Object: CLSID_SLTMVideoPlayer

The given interface allows select up to 6 live video inputs when you select command ShowLive of the video player (interface ISLTMVideoPlayer). Methods of the given interface allow operate with logic video inputs. Therefore before the beginning of work it is necessary to configure and enable the inputs (see notes 1, 2).

HRESULT ResetAllSettings() – restore of default settings for the FD300 board (the state was configured by FDConfig application).

HRESULT SaveLiveSet([in] IUnknown *pPropBag) – writes all configured settings into the specified container (standard property bag interface).

HRESULT LoadLiveSet([in] IUnknown *pPropBag) – reads configuration from the specified container and try to apply the setting.

HRESULT ConfigureLive([in] SLTMVPVideoLineType nLiveIndex, [in] long nHWND) – show configuration dialog for specified logical video input nLiveIndex. The dialog will be a child of specified window HWND nHWND.

HRESULT get_Name([in] SLTMVPVideoLineType nLiveIndex, [out, retval] BSTR *pName) – returns the user name of configured video input with specified index nLineIndex.

HRESULT get_EnableLive([in] SLTMVPVideoLineType nLiveIndex, [out, retval] long *pEnabled) – returns 1 if the specified video input nLineIndex was enabled for usage.

HRESULT put_EnableLive([in] SLTMVPVideoLineType nLiveIndex, [in] long bEnable) – enable/disable of usage the specified video input nLineIndex.

HRESULT ApplyEnables() – try to lock resources for usage all enabled video inputs. Attention: you have to call the method after you enable/disable usage of video inputs.

HRESULT PrepareLive([in] SLTMVPVideoLineType nLiveIndex) – load settings of specified video input nLiveIndex into inactive video decoder of FD300 board. The specified video input will be showed on next command ShowLive of the video player (see interface ISLTMVideoPlayer).

Notes:

- FD300 board has 2 video decoder. Every time one of the decoders is active and another is inactive. Video from the active decoder can be showed on FD300 video output. To make transition from current video to any preconfigured LIVE video input you need call method PrepareLive of the interface ISLTMLive4VideoPlayer with the index of corresponding logical video input and call method ShowLive of the interface ISLTMVideoPlayer. You have to wait small time before real video switching for video decoder synchronization. It may be 5-10 frames. You can switch from one LIVE video to another LIVE video if you don’t use genlock mode. In genlock mode you have to switches to computer video between two LIVE video switching.

- Type SLTMVPVideoLineType defines type of a logic video input:

SLTMVPVideo_Live1 = 1	 video input #1

SLTMVPVideo_Live2 = 2	 video input #2

SLTMVPVideo_Live3 = 3	 video input #3

SLTMVPVideo_Live4 = 4	 video input #4

SLTMVPVideo_Live5 = 5	 video input #5

SLTMVPVideo_Live6 = 6	 video input #6

SLTMVPVideo_Live4Genlock = -1	 video input for genlock

Last video input index used for genlock enabling. You cannot use the index SLTMVPVideo_Live4Genlock as parameter of method PrepareLive. The index SLTMVPVideo_Live4Genlock can be used with enabling & configuration methods (put_EnableLive & ConfigureLive) only for genlock or master mode selection.

## ISLTMAudio4VideoPlayer

Object: CLSID_SLTMVideoPlayer

The given interface allows control parameters of audio part of FD300 board (loudness and mute) (interface ISLTMVideoPlayer). Methods of the given interface allow operating not physical audio inputs/outputs but logic audio lines. Therefore before the beginning of work it is necessary to configure the logical audio inputs and assign it to real physical lines (sound inputs/outputs of FD300 board). See notes 1, 2.

HRESULT get_DeviceCount ([out, retval] long *pVal) – returns number of FD300 audio devices, which was installed in the system.

HRESULT get_DeviceName ([in] long DeviceIndex, [out, retval] BSTR *pName) – returns a text name of the sound device with specified index DeviceIndex.

HRESULT SelectDevice ([in] long DeviceIndex) – chooses the sound device with specified index DeviceIndex for a audio playback. Attention: it is necessary to call this function before call of function AcquireBoard of interface ISLTMVideoPlayer!

HRESULT get_LineCount ([out, retval] long *pVal) – returns number of input audio lines for the selected sound device. It should be 9 (3 stereo-inputs and 6 mono-inputs). See the note 1.

HRESULT get_LineName ([in] long LineIndex, [out, retval] BSTR *pName) – returns the text name of an input audio line with specified index LineIndex for the current sound device.

HRESULT AssignLine ([in] SLTMVPAudioVolumeType LineType, [in] long LineIndex) – assigns a physical input with specified index LineIndex to logic line LineType. See the note 1.

HRESULT ReadPicks () – reads out the current peak values of audio volume for all lines. Only after a call of this function it is possible to receive real values of volume of audio signal in a line with the help of calls of following function GetLinePicks.

HRESULT GetLinePicks ([in] SLTMVPAudioVolumeType LineType, [out] double *pLeft, [out] double *pRight) – returns the current peak value of volume in left and right channels (in decibels) for the specified logic sound line. See notes 2, 3.

HRESULT ReadVolumes () – reads out the current values of audio volume of all sound lines. Only after a call of this function it is possible to receive real values of audio volume with the help of calls of the following functions get_Volume, get_Mute.

HRESULT get_Volume ([in] SLTMVPAudioVolumeType LineType, [out, retval] double *pVal) – returns the current value of a level of volume for logic line LineType in decibels (-96.0 … 12.0).

HRESULT put_Volume ([in] SLTMVPAudioVolumeType LineType, [in] double newVal) – sets value of a level of volume for logic line LineType in decibels (-96.0 … 12.0).

HRESULT get_Enable ([in] SLTMVPAudioVolumeType LineType, [out, retval] long *pVal) – returns value 1 into pVal if the specified audio line LineType is enabled for usage (the line is active), and returns 0 if the sound is not enabled (the line is inactive). It is used usually for a computer sound line (SLTMVPAudio_OutputVolume). See the note 4.

HRESULT put_Enable ([in] SLTMVPAudioVolumeType LineType, [in] long newVal) – value 1 of parameter pVal enables audio playback for specified line LineType, value 0 disable audio playback. It is used usually for microphone/linear inputs(entrances). See. The note 4.

HRESULT get_Mute ([in] SLTMVPAudioVolumeType LineType, [out, retval] long *pVal) – returns in pVal value 1 if on specified sound line LineType the sound is forbidden (a line заглушена), and 0 if the sound it is resolved(allowed) (a line it it is resolved(allowed)). See. The note 4.

HRESULT put_Mute ([in] SLTMVPAudioVolumeType LineType, [in] long newVal) – value of 1 parameter pVal forbids sound line LineType (the sound on a line is switched off), value 0 – allows (a sound on a line it is resolved(allowed)). See. The note 4.

Notes:

- Each FD300 board has six physical audio mono inputs and six mono outputs. Each pair of mono inputs is incorporated as stereo input. The given interface allows operating volume not a physical but a logic audio line (see the note 2). Therefore before the beginning of work it is necessary to assign physical inputs to logic lines with the help of method AssignLine of the given interface.

The current stereo output selected by function SelectDevice. After that it is necessary to assign all required physical audio inputs AssignLine. The following indexes of physical audio lines are used:

| Index of a physical sound line | Assignment |
| --- | --- |
| 0 | Stereo-line 0 |
| 1 | Stereo-line 1 |
| 2 | Stereo-line 2 |
| 3 | Mono-line 0 (the left channel of a stereo-line 0) |
| 4 | Mono-line 1 (the right channel of a stereo-line 0) |
| 5 | Mono-line 2 (the left channel of a stereo-line 1) |
| 6 | Mono-line 3 (the right channel of a stereo-line 1) |
| 7 | Mono-line 4 (the left channel of a stereo-line 2) |
| 8 | Mono-line 5 (the right channel of a stereo-line 2) |

- Type SLTMVPAudioVolumeType defines type of logic audio line:

SLTMVPAudio_OutputVolume = 0 	a computer sound (for example, from a file)

SLTMVPAudio_LiveVideo1 = 1	a sound going together with taking place video #1

SLTMVPAudio_LiveVideo2 = 2	a sound going together with taking place video #2

SLTMVPAudio_LiveVideo3 = 3	a sound going together with taking place video #3

SLTMVPAudio_LiveVideo4 = 4	a sound going together with taking place video #4

SLTMVPAudio_LiveVideo5 = 5	a sound going together with taking place video #5

SLTMVPAudio_LiveVideo6 = 6	a sound going together with taking place video #6

SLTMVPAudio_LiveAux = 11	additional audio channel

SLTMVPAudio_None = -1	not used

Assignment of real physical input of sound line of FD300 board to logic lines is made with the help of method AssignLine of the given interface (see the note 1). It is not possible to assign any physical input lines to logic line SLTMVPAudio_OutputVolume. This line is used for volume and mute adjustment of audio from files (movie).

- To obtain information of the current levels of volumes and peaks for sound lines at first you have to call ReadPicks or ReadVolumes and then call GetLinePicks or get_Volume/get_Mute. It was done to increase performance of sequential calls of GetLinePicks or get_Volume/get_Mute.

- The Mute property (prohibition of the line usage) has higher priority, than Enable property (enabling of the line usage). For example, if for any line property Enable is established in 1, and Mute too in 1 the sound on the line will be forbidden. Also property Mute corresponds to real flag Mute on inputs/outputs of the FD300 audio device.

## ISLTMForegroundPlayer

Object: CLSID_SLTMForegroundPlayer

The given interface is intended for "old style" titles playback. Now we recommend to use Title SDK instead of the interface usage. It does not support audio playback. You have to use additional audio player for synchronous audio playback.

HRESULT get_BoardsCount ([out, retval] long *pVal) – returns number of FD300 boards, which was installed in the system.

HRESULT get_BoardName ([in] long BoardIndex, [out, retval] BSTR *pName) – returns the text name of FD300 board with specified index BoardIndex.

HRESULT UsedMemorySizeMB ([in] long use_MB) – sets a memory size, used for video buffering (recommended value: 60). Attention: this function should be called before call of function AcquireBoard.

HRESULT AcquireBoard ([in] long BoardIndex) – tries to lock resources playback video on specified FD300 board (with index BoardIndex, started from zero).

HRESULT FreeBoard () – releases all locked resources.

HRESULT GetBoardInfo ([out] long *width, [out] long *height, [out] double *frame_rate) – returns the current parameters of the selected FD300 board: width – width of the screen (in pixels), height – height of the screen (in pixels), frame_rate – video frequency (in frames per second).

HRESULT LoadLogo ([in] long logo_index, [in] BSTR file_name) – Not supported.

HRESULT ShowLogo ([in] long logo_index, [in] double start_time, [in] double crossfade_duration) – Not supported.

HRESULT HideLogo ([in] double start_time, [in] double crossfade_duration) – Not supported.

HRESULT Clear ([in] double start_time, [in] double crossfade_duration) – will hide all previously shown titles at specified global time start_time with specified fade-out duration crossfade_duration (in seconds). See the note 1.

HRESULT LoadImage ([in] long image_index, [in] BSTR file_name) – Not supported.

HRESULT ShowImage ([in] long image_index, [in] double start_time, [in] double crossfade_duration) – Not supported.

HRESULT PrepareVideo ([in] BSTR file_name, [out] double *duration) – loads and prepares for reproduction a AVI-file or a graphic file (in format JPEG, Bitmap, Targa or PNG) with specified file_name. Duration of prepared video will be returned in parameter duration.

HRESULT PlayVideo ([in] double start_time, [in] double crossfade_duration) – will start playback of prepared file at specified global time start_time with the specified duration crossfade_duration (in seconds) of cross-fade transition. See the note 1.

HRESULT PauseVideo () – pause video playback or show the first frame of prepared video.

HRESULT UnpauseVideo () – continues playback of paused video.

HRESULT get_RestDuration ([out] double *pEndingTime, [out, retval] double *pRestDuration) – returns planned time of the playback finish into parameter pEndingTime and time up to the end of crossfade finish into parameter pRestDuration (in seconds). See the note 1.

HRESULT get_PrepareState ([out, retval] long *pVal) – returns the current state after last command PrepareVideo. See the note 2.

HRESULT get_ErrorDescription ([out, retval] BSTR *pVal) – returns a string with the text description of the error of last video preparing or playback.

HRESULT ResetPrepare () – releases the resources seized by playback preparing (closes the file, free the buffers).

Notes:

- Global time (in seconds) is an internal time of FD300 board. It starts from the FD300 board initiation. Crossfade – transition from one video-image on another through the cross-fade effect.

- For playback title the same logic is used as well as at work with interface ISLTMVideoPlayer (see the note 2 to the description of interface ISLTMVideoPlayer).

- Supported video formats of files are listed in the description of interface ISLTMPlayMovie, and graphic formats of images – in description ISLTMPlayImage.

- The object does not allow to playback more then one title simultaneously.

## ISLTMAudioPlayer

Object: CLSID_SLTMAudioPlayer

The given interface is intended for playback of audio files.

HRESULT AttachToVideoPlayer ([in] IUnknown *pVideoPlayer) – on this command sound player obtain current audio settings from the specified video player (FD300 board index and index of audio output line), and then open the same sound device.

HRESULT PrepareAudio ([in] BSTR file_name, [out] double *duration) – loads and prepares for playback audio file with specified file name. Duration of prepared audio will be returned in parameter duration.

HRESULT PlayAudio ([in] double start_time, [in] double crossfade_duration) – will start playback of prepared before audio file at specified global time start_time with the specified duration crossfade_duration (in seconds) of cross-fade effect. See the note 1.

HRESULT StopAudio ([in] double start_time, [in] double crossfade_duration) – will stop playback of the audio file at specified global time start_time with the specified duration crossfade_duration (in seconds) of fade-out effect. See the note 1.

HRESULT PauseAudio () – pause playback of played audio file.

HRESULT UnpauseAudio () – continues playback of paused audio file.

HRESULT get_RestDuration ([out] double *pEndingTime, [out, retval] double *pRestDuration) – returns planned time of the playback finish into parameter pEndingTime and time up to the end of crossfade finish into parameter pRestDuration (in seconds). See the note 1.

HRESULT get_PrepareState ([out, retval] long *pVal) – returns the current state after last command PrepareAudio. See the note 2.

HRESULT get_ErrorDescription ([out, retval] BSTR *pVal) – returns a string with the text description of the error of last video preparing or playback.

HRESULT ResetPrepare () – releases the resources seized by playback preparing (closes the file, free the buffers).

HRESULT get_Volume ([out, retval] double *pVal) – returns the current level of volume for playback of audio file (in decibels).

HRESULT put_Volume ([in] double newVal) – sets a level of volume for playback of audio file (in decibels). Allowable values of volume: from -96 up to +12, and at a level of volume -96 sounds will not be audible absolutely, at a zero level of volume the sound will be is reproduced with nominal loudness, and at a level of loudness +12 sound will become stronger in 4 times.

HRESULT get_Enable ([out, retval] long *pVal) – returns a condition of "enabling" of playback of audio file. See the note 3.

HRESULT put_Enable ([in] long newVal) – sets a condition of "enabling" of playback of audio file. See the note 3.

HRESULT get_Mute ([out, retval] long *pVal) – returns a condition of "mute" of playback of audio file. See the note 3.

HRESULT put_Mute ([in] long newVal) – establishes a condition of "mute" of playback of audio file. See the note 3.

Notes:

- Global time (in seconds) is an internal time of FD300 board. It starts from the FD300 board initiation. Crossfade – transition from one audio file on another through the cross-fade effect.

- For playback audio files the same logic is used as well as at work with interface ISLTMVideoPlayer (see the note 2 to the description of interface ISLTMVideoPlayer).

- The Mute property (prohibition) has higher priority, than Enable property. For example, if property Enable is established in 1, and Mute too in 1 the output volume will be forbidden (the sound will be not audible).

# The appendix

## Example: reproduction of sequence of AVI-files with use of object SLTMVideoPlayer

Object SLTMVideoPlayer allows to playback sequence of different videos and audio files, for example, sequence AVI- and MPEG-files (with or without audio). For playback of sequence of AVI-files with the help of object SLTMVideoPlayer it is necessary to make the following steps:

- To create object SLTMVideoPlayer and to select current FD300 board (function AcquireBoard).

- To prepare for playback an AVI-file (to start preparing and to wait the finish of the preparing).

- To define time of playback start (for example, as time of the finishing a playback of previous clip minus time of the transition).

- To start playback of the prepared AVI-file (to start playback and to wait the transition finishing).

- To obtain time of the playback finish from video player and to calculate time of start of the following movie, and then to repeat steps 2 and 4.

Strongly recommended to check result of each operation (HRESULT) with the help macro FAILED and to stop execution if the call of any method has returned any error.

Below it is given an example the procedure making all listed operations. The file names of AVI-files on a local disk is placed in parameter pstrFileName; number of files – in parameter nFileNumber; the parameter fTransition sets duration of cross-fade between files:

HRESULT PlayList (LPCTSTR pstrFileName [], int nFileNumber, double fTransition)

{

// create object SLTMVideoPlayer

CComPtr <ISLTMVideoPlayer> pPlayer;

HRESULT hr = pPlayer. CoCreateInstance (CLSID_SLTMVideoPlayer);

if (FAILED (hr)) return hr;

// obtain number of FD300 boards

long nb;

hr = pPlayer-> get_BoardsCount (*nb);

if (FAILED (hr)) return hr;

// if there is no boards then return a error

if (nb <1) return E_FAIL;

//  select first FD300 board (with index 0)

hr = pPlayer-> AcquireBoard (0);

if (FAILED (hr)) return hr;

//  select start time – "right now"

double fStartTime = 0;

//  cycle by  AVI-files

for (int i=0; i <nFileNumber; i ++)

{

//  obtain the name of a file in format BSTR

CComBSTR bFileName (pstrFileName [i]);

//  start video preparing

double duration;

hr = pPlayer-> PrepareVideo (bFileName, *duration);

if (FAILED (hr)) return hr;

//  wait, while the file will be prepared

for (;;)

{

// check the preparing state

long state;

hr = pPlayer-> get_PrepareState (*state);

if (FAILED (hr)) return hr;

if (state == SLTMVideoPlayerPreparedVideo) break;

else if (state == SLTMVideoPlayerPreparingVideo) continue;

else if (state == SLTMVideoPlayerWasError) return E_INVALIDARG;

else return E_UNEXPECTED;

// it is not ready – wait 100 ms

sleep (100);

}

// the file is ready for playback – start playback at time start

hr = pPlayer-> PlayVideo (fStartTime, fTransition);

if (FAILED (hr)) return hr;

//  wait the finish of transition

for (;;)

{

// check the status of transition

long state;

hr = pPlayer-> get_PrepareState (*state);

if (FAILED (hr)) return hr;

if (state == SLTMVideoPlayerStandby) break;

else if (state == SLTMVideoPlayerInTransition) {}

else if (state == SLTMVideoPlayerWasError) return E_INVALIDARG;

else return E_UNEXPECTED;

// it is not ready – wait 100 ms

sleep (100);

}

// the file is normally played – obtain the time of playback finish

hr = pPlayer-> get_RestDuration (*fStartTime, *duration);

if (FAILED (hr)) return hr;

// calculate start time for next file – take away transition time

fStartTime - = fTransition;

}

// here still it is necessary to wait the finish of last file

for (;;)

{

double fEndTime, fRestTime;

hr = pPlayer-> get_RestDuration (*fEndTime, *fRestTime);

if (FAILED (hr)) return hr;

if (fRestTime <= 0.01) break;

// need wait little bit more

sleep (100);

}

// release resources

pPlayer-> FreeBoard ();

// all  done

return S_OK;

}